GNS3 — Network Project Documentation

Complete Documentation: Topology, Configuration & Testing

# 1. Topology & Connections

## 1.1 Connection Diagram

| From | Port | To | Port |
| --- | --- | --- | --- |
| PC1-IT (VPCS) | eth0 | ESW1 | Fa1/0 |
| PC2-HR (VPCS) | eth0 | ESW1 | Fa1/1 |
| PC3-GUESTS (VPCS) | eth0 | ESW2 | Fa1/0 |
| ESW1 | Fa1/2 | ESW2 | Fa1/2 |
| R1-CE | Fa0/0 | ESW1 | Fa1/3 |
| R1-CE | Serial1/0 | R2-ISP | Serial1/0 |
| R2-ISP | Fa0/0 | NAT1 | nat0 |
| Ubuntu-Server | eth0 (enp0s3) | ESW2 | Fa1/3 |

## 1.2 VLAN Plan

| VLAN | Name | Subnet | Device |
| --- | --- | --- | --- |
| 10 | IT | 192.168.10.0/24 | PC1-IT -> ESW1 Fa1/0 |
| 20 | HR | 192.168.20.0/24 | PC2-HR -> ESW1 Fa1/1 |
| 30 | GUEST | 192.168.30.0/24 | PC3-GUESTS -> ESW2 Fa1/0 |
| 40 | SERVERS | 192.168.40.0/24 | Ubuntu-Server -> ESW2 Fa1/3 |

# 2. ESW1 Configuration

## 2.1 VLANs & Ports

! Initialization

enable

configure terminal

hostname SW1

! Create VLANs

vlan 10

name IT

vlan 20

name HR

vlan 30

name GUEST

vlan 40

name SERVERS

! PC1 -> VLAN 10 (Fa1/0)

interface FastEthernet1/0

switchport mode access

switchport access vlan 10

no shutdown

! PC2 -> VLAN 20 (Fa1/1)

interface FastEthernet1/1

switchport mode access

switchport access vlan 20

no shutdown

! TRUNK -> ESW2 (Fa1/2)

interface FastEthernet1/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

no shutdown

! TRUNK -> Router CE (Fa1/3)

interface FastEthernet1/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

no shutdown

! Save

end

write memory

# 3. ESW2 Configuration

## 3.1 VLANs & Ports

| Port | Connected To | VLAN |
| --- | --- | --- |
| Fa1/0 | PC3-GUESTS | 30 |
| Fa1/2 | ESW1 (trunk) | 10,20,30,40 |
| Fa1/3 | Ubuntu Server | 40 |

! Initialization

enable

configure terminal

hostname SW2

! Create VLANs

vlan 10

name IT

vlan 20

name HR

vlan 30

name GUEST

vlan 40

name SERVERS

! PC3 -> VLAN 30 (Fa1/0)

interface FastEthernet1/0

switchport mode access

switchport access vlan 30

no shutdown

! Ubuntu Server -> VLAN 40 (Fa1/3)

interface FastEthernet1/3

switchport mode access

switchport access vlan 40

no shutdown

! TRUNK -> ESW1 (Fa1/2)

interface FastEthernet1/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

no shutdown

! Save

end

write memory

# 4. Router CE Configuration (R1-CE)

## 4.1 Sub-interfaces, OSPF, NAT, ACL

! Initialization

enable

configure terminal

hostname CE

! Physical interface toward SW1

interface FastEthernet0/0

no ip address

no shutdown

! VLAN 10 - IT

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.40.10

ip nat inside

! VLAN 20 - HR

interface FastEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.40.10

ip nat inside

! VLAN 30 - GUEST

interface FastEthernet0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

ip helper-address 192.168.40.10

ip nat inside

! VLAN 40 - SERVERS

interface FastEthernet0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

ip nat inside

! Serial link toward ISP

interface Serial1/0

ip address 220.110.0.1 255.255.255.252

clock rate 64000

ip nat outside

no shutdown

! OSPF

router ospf 1

router-id 1.1.1.1

network 220.110.0.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.40.0 0.0.0.255 area 0

! NAT ACL

access-list 10 permit 192.168.10.0 0.0.0.255

access-list 10 permit 192.168.20.0 0.0.0.255

access-list 10 permit 192.168.30.0 0.0.0.255

access-list 10 permit 192.168.40.0 0.0.0.255

! PAT

ip nat inside source list 10 interface Serial1/0 overload

! Default route

ip route 0.0.0.0 0.0.0.0 220.110.0.2

! GUEST ACL - network isolation

ip access-list extended GUESTS_ACL

permit udp any any eq bootps

permit udp any any eq bootpc

permit udp any host 192.168.40.10 eq 53

permit tcp any host 192.168.40.10 eq 53

permit tcp any host 192.168.40.10 eq 80

deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255

deny ip 192.168.30.0 0.0.0.255 192.168.40.0 0.0.0.255

permit ip any any

exit

! Apply GUEST ACL

interface FastEthernet0/0.30

ip access-group GUESTS_ACL in

! HR ACL - isolation from IT

ip access-list extended HR_ACL

deny ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip any any

exit

! Apply HR ACL

interface FastEthernet0/0.20

ip access-group HR_ACL in

! Save

end

write memory

# 5. Router ISP Configuration (R2-ISP)

! Initialization

enable

configure terminal

hostname ISP

! Serial link toward CE

interface Serial1/0

ip address 220.110.0.2 255.255.255.252

no shutdown

! Interface toward GNS3 NAT Node

interface FastEthernet0/0

ip address dhcp

no shutdown

! Default route

ip route 0.0.0.0 0.0.0.0 FastEthernet0/0

! OSPF

router ospf 1

router-id 2.2.2.2

network 220.110.0.0 0.0.0.3 area 0

! Save

end

write memory

# 6. Ubuntu Server Configuration

## 6.1 Static IP

sudo nano /etc/netplan/00-installer-config.yaml

# File content:

network:

version: 2

ethernets:

enp0s3:

dhcp4: false

addresses:

- 192.168.40.10/24

routes:

- to: default

via: 192.168.40.1

nameservers:

addresses:

- 8.8.8.8

- 1.1.1.1

# Apply configuration

sudo chmod 600 /etc/netplan/00-installer-config.yaml

sudo netplan apply

## 6.2 DHCP Server

sudo apt update

sudo apt install isc-dhcp-server -y

sudo nano /etc/default/isc-dhcp-server

# Set:

INTERFACESv4="enp0s3"

sudo nano /etc/dhcp/dhcpd.conf

# Add:

authoritative;

subnet 192.168.10.0 netmask 255.255.255.0 {

range 192.168.10.100 192.168.10.150;

option routers 192.168.10.1;

option domain-name-servers 192.168.40.10;

}

subnet 192.168.20.0 netmask 255.255.255.0 {

range 192.168.20.100 192.168.20.150;

option routers 192.168.20.1;

option domain-name-servers 192.168.40.10;

}

subnet 192.168.30.0 netmask 255.255.255.0 {

range 192.168.30.100 192.168.30.130;

option routers 192.168.30.1;

option domain-name-servers 192.168.40.10;

}

sudo systemctl restart isc-dhcp-server

sudo systemctl enable isc-dhcp-server

sudo systemctl status isc-dhcp-server

## 6.3 DNS Server (BIND9)

sudo apt install bind9 -y

sudo nano /etc/bind/named.conf.local

# Add firma.local zone:

zone "firma.local" {

type master;

file "/etc/bind/db.firma.local";

};

sudo cp /etc/bind/db.local /etc/bind/db.firma.local

sudo nano /etc/bind/db.firma.local

# Add A record:

www     IN      A       192.168.40.10

sudo systemctl restart bind9

# 7. Testing Steps

## 7.1 Testing Checklist

| # | Where | Command | Expected Result |
| --- | --- | --- | --- |
| 1 | ISP Router | show ip interface brief | Fa0/0 up/up, Serial1/0 up/up |
| 2 | ISP Router | ping 8.8.8.8 | 5/5 packets — internet OK |
| 3 | CE Router | show ip ospf neighbor | ISP neighbor visible |
| 4 | CE Router | show ip nat translations | Private IP -> 220.110.0.1 |
| 5 | Ubuntu Server | ping 192.168.40.1 | VLAN40 gateway OK |
| 6 | Ubuntu Server | ping 8.8.8.8 | Full internet OK |
| 7 | PC1-IT (VPCS) | dhcp | IP 192.168.10.x received |
| 8 | PC2-HR (VPCS) | dhcp | IP 192.168.20.x received |
| 9 | PC3-GUESTS (VPCS) | dhcp | IP 192.168.30.x received |
| 10 | PC3-GUESTS | ping 192.168.20.1 | DENIED - ACL working |
| 11 | PC (browser) | http://192.168.40.10 | Ubuntu web page loads |

GNS3 Network Project — Complete Documentation