GNS3 — Network Project Documentation

Complete Documentation: Topology, Configuration & Testing

# 1. Topology & Connections

## 1.1 Connection Diagram

| From | Port | To | Port | VLAN / Network Type | Note / Function in Project |
| --- | --- | --- | --- | --- | --- |
| PC1 | eth0 | ESW1 | f1/0 | VLAN 10 (IT) | IT network user |
| PC2 | eth0 | ESW1 | f1/1 | VLAN 20 (HR) | HR network user |
| Kali Linux | enp0s3 | ESW1 | f1/4 | VLAN 10 (IT) | Test / security station on the IT network |
| ESW1 | f1/2 | ESW2 | f1/2 | Trunk (10, 20, 30, 40) | Inter-switch link (trunking) |
| ESW1 | f1/3 | R1 | f0/0 | Trunk (10, 20, 30, 40) | Router-switch link (router-on-a-stick) |
| PC3 | eth0 | ESW2 | f1/0 | VLAN 30 (GUEST) | Guest network user |
| Ubuntu Server | enp0s3 | ESW2 | f1/3 | VLAN 40 (SERVERS) | Central server (BIND9 DNS + ISC-DHCP) |
| R1 | f2/0 | R2 | f0/0 | IP transit (220.110.0.0/30) | WAN link / routing to the ISP |
| R2 | f2/1 | NAT1 (Cloud) | DHCP interface | Public external traffic | Final, secured exit to the Internet |

## 1.2 VLAN Plan

| VLAN | Name | Subnet | Device(s) |
| --- | --- | --- | --- |
| 10 | IT | 192.168.10.0/24 | PC1 -> ESW1 f1/0; Kali Linux -> ESW1 f1/4 |
| 20 | HR | 192.168.20.0/24 | PC2 -> ESW1 f1/1 |
| 30 | GUEST | 192.168.30.0/24 | PC3 -> ESW2 f1/0 |
| 40 | SERVERS | 192.168.40.0/24 | Ubuntu Server -> ESW2 f1/3 |

# 2. ESW1 Configuration

## 2.1 VLANs & Ports

enable

configure terminal

hostname ESW1

vlan 10

name IT

vlan 20

name HR

vlan 30

name GUEST

vlan 40

name SERVERS

exit

! Port f1/4 connected to Kali Linux (VLAN 10)

interface FastEthernet 1/4

switchport mode access

switchport access vlan 10

spanning-tree portfast

! Port f1/0 connected to PC1 (VLAN 10)

interface FastEthernet 1/0

switchport mode access

switchport access vlan 10

spanning-tree portfast

! Port f1/1 connected to PC2 (VLAN 20)

interface FastEthernet 1/1

switchport mode access

switchport access vlan 20

spanning-tree portfast

! Port f1/2 connected to Switch ESW2 (Trunk)

interface FastEthernet 1/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

! Port f1/3 connected to Router R1 f0/0 (Trunk)

interface FastEthernet 1/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

exit

# 3. ESW2 Configuration

## 3.1 VLANs & Ports

| Port | Connected To | VLAN |
| --- | --- | --- |
| f1/0 | PC3-GUESTS | 30 |
| f1/2 | ESW1 (trunk) | 10,20,30,40 |
| f1/3 | Ubuntu Server | 40 |

enable

configure terminal

hostname ESW2

vlan 10

name IT

vlan 20

name HR

vlan 30

name GUEST

vlan 40

name SERVERS

exit

! Port f1/0 connected to PC3 (VLAN 30)

interface FastEthernet 1/0

switchport mode access

switchport access vlan 30

spanning-tree portfast

! Port f1/3 links to the Ubuntu Server (VLAN 40)

interface FastEthernet 1/3

switchport mode access

switchport access vlan 40

spanning-tree portfast

! Port f1/2 connected back to ESW1 f1/2 (Trunk)

interface FastEthernet 1/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

exit

# 4. R1 Router Configuration

## 4.1 Sub-interfaces, OSPF, NAT, ACL

enable

configure terminal

hostname R1

! Bringing up the physical LAN interface toward the switch (port f0/0, per diagram)

interface FastEthernet 0/0

no shutdown

! Sub-interface configuration (inter-VLAN routing & DHCP relay)

interface FastEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.40.10

ip nat inside

interface FastEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.40.10

ip nat inside

interface FastEthernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

ip helper-address 192.168.40.10

ip nat inside

interface FastEthernet 0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

ip nat inside

exit

! Real WAN interface configuration toward ISP router R2 (port f2/0, per diagram)

interface FastEthernet 2/0

ip address 220.110.0.1 255.255.255.252

ip nat outside

no shutdown

exit

! UDP FIX: UDP/DNS translations expire automatically after 10 seconds (avoids stale NAT entries after a restart)

ip nat translation udp-timeout 10

! Access list and NAT overload configuration on the correct interface

ip access-list standard NAT_ACL

permit 192.168.10.0 0.0.0.255

permit 192.168.20.0 0.0.0.255

permit 192.168.30.0 0.0.0.255

permit 192.168.40.0 0.0.0.255

exit

ip nat inside source list NAT_ACL interface FastEthernet 2/0 overload

! OSPF configuration (routing toward R2)

router ospf 1

network 220.110.0.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.40.0 0.0.0.255 area 0

exit

! ----- Security access control lists (ACL) -----

! GUEST_ACL: allows only Web (port 80) to the Server, blocks ICMP and access to other VLANs

ip access-list extended GUEST_ACL

permit tcp 192.168.30.0 0.0.0.255 host 192.168.40.10 eq www

deny icmp 192.168.30.0 0.0.0.255 host 192.168.40.10

deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255

permit ip any any

exit

! HR_ACL: blocks the HR network's access to the IT network

ip access-list extended HR_ACL

deny ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip any any

exit

! Applying the ACLs inbound on the sub-interfaces

interface FastEthernet 0/0.30

ip access-group GUEST_ACL in

interface FastEthernet 0/0.20

ip access-group HR_ACL in

exit

! Permanently saving R1's configuration

write memory

# 5. R2 Router Configuration

enable

configure terminal

hostname R2

! Interface connected to router R1 (port f0/0, per diagram)

interface FastEthernet 0/0

ip nat inside

no shutdown

exit

! WAN interface connected to Cloud NAT1 (port f2/1, per diagram)

interface FastEthernet 2/1

ip address dhcp

ip nat outside

no shutdown

exit

! UDP FIX: UDP sessions also expire quickly on the ISP router, so port 53 doesn't get blocked

ip nat translation udp-timeout 10

! NAT configuration on the ISP router, adapted for a dynamic IP (interface f2/1)

ip access-list standard ISP_NAT

permit 220.110.0.0 0.0.0.3

exit

ip nat inside source list ISP_NAT interface FastEthernet 2/1 overload

! OSPF configuration + automatic default-route advertisement toward R1

router ospf 1

network 220.110.0.0 0.0.0.3 area 0

default-information originate

exit

! Permanently saving R2's configuration

write memory

# 6. Ubuntu Server Configuration

## 6.1 Static IP

sudo nano /etc/netplan/00-installer-config.yaml

# File content:

network:

version: 2

ethernets:

enp0s3:

dhcp4: false

addresses:

- 192.168.40.10/24

routes:

- to: default

via: 192.168.40.1

nameservers:

addresses:

- 127.0.0.1

# Apply configuration

sudo chmod 600 /etc/netplan/00-installer-config.yaml

sudo netplan apply

## 6.2 DHCP Server

sudo apt update

sudo apt install isc-dhcp-server -y

sudo nano /etc/default/isc-dhcp-server

# Set:

INTERFACESv4="enp0s3"

sudo nano /etc/dhcp/dhcpd.conf

# Add:

authoritative;

subnet 192.168.10.0 netmask 255.255.255.0 {

range 192.168.10.100 192.168.10.150;

option routers 192.168.10.1;

option domain-name-servers 192.168.40.10;

}

subnet 192.168.20.0 netmask 255.255.255.0 {

range 192.168.20.100 192.168.20.150;

option routers 192.168.20.1;

option domain-name-servers 192.168.40.10;

}

subnet 192.168.30.0 netmask 255.255.255.0 {

range 192.168.30.100 192.168.30.130;

option routers 192.168.30.1;

option domain-name-servers 192.168.40.10;

}

sudo systemctl restart isc-dhcp-server

sudo systemctl enable isc-dhcp-server

sudo systemctl status isc-dhcp-server

## 6.3 DNS Server (BIND9)

sudo apt install bind9 -y

sudo nano /etc/bind/named.conf.options

# Set global resolver options (recursion + forwarders):

options {

directory "/var/cache/bind";

recursion yes;

allow-recursion { any; };

allow-query { any; };

forwarders {

8.8.8.8;

1.1.1.1;

};

dnssec-validation auto;

listen-on { any; };

listen-on-v6 { any; };

};

sudo nano /etc/bind/named.conf.local

# Add firma.local zone:

zone "firma.local" {

type master;

file "/etc/bind/db.firma.local";

};

sudo cp /etc/bind/db.local /etc/bind/db.firma.local

sudo nano /etc/bind/db.firma.local

# Add A record:

www     IN      A       192.168.40.10

sudo systemctl restart bind9

# 7. Testing Steps

## 7.1 Testing Checklist

| # | Where | Command | Expected Result |
| --- | --- | --- | --- |
| 1 | R2 Router | show ip interface brief | Fa0/0 up/up, Fa2/1 up/up |
| 2 | R2 Router | ping 8.8.8.8 | 5/5 packets — internet OK |
| 3 | R1 Router | show ip ospf neighbor | R2 neighbor visible |
| 4 | R1 Router | show ip nat translations | Private IP -> 220.110.0.1 |
| 5 | Ubuntu Server | ping 192.168.40.1 | VLAN 40 gateway OK |
| 6 | Ubuntu Server | ping 8.8.8.8 | Full internet OK |
| 7 | PC1 (VPCS) | dhcp | IP 192.168.10.x received |
| 8 | PC2 (VPCS) | dhcp | IP 192.168.20.x received |
| 9 | PC3 (VPCS) | dhcp | IP 192.168.30.x received |
| 10 | Kali Linux | ip a / ping 192.168.10.1 | IP in 192.168.10.x range, gateway reachable |
| 11 | PC3 (GUEST) | ping 192.168.20.1 | DENIED — ACL working |
| 12 | PC (browser) | http://192.168.40.10 | Ubuntu web page loads |

GNS3 Network Project — Complete Documentation